--------------------------------------------------------------------------------

--

--   AM2901 Benchmark

--

-- Source: AMD data book

--

-- VHDL Benchmark author Indraneel Ghosh

--                       University Of California, Irvine, CA 92717

--

-- Developed on Nov 1, 1991

--

--------------------------------------------------------------------------------

THE Functional Block MODEL

The chip has been partitioned into functional blocks which are

modeled by VHDL alogorithmic Behaviour

The blocks are :

** alu_inputs : ALU source selector

** alu : ALU

** mem : RAM

** Q_reg : Q register

** output_and_shifter : output selector and the shifter outputs from RAM and

Q-register.

Guarded concurrent blocks "Q_reg" and "mem" are used to represent the

sequential elements in the chip. These blocks are trigerred by the clock.

Both the guarded blocks have a master-slave configuration.

-----------------------------------------------------------------

VARIABLES, SIGNALS AND TYPES :

File "am2901.doc" contains a general description of the chip and its ports.

This model uses the following global and local signals and variables:

*******************************************************

GLOBAL SIGNALS :

RAM ==> The 16-word by 4-bit RAM.

RE ==> Input data for ALU, selected by ALU data selector. ( 4 bits)

S ==> Input data for ALU, selected by ALU data selector. ( 4 bits)

A ==> The RAM word addressed by Aadd is read into this variable. ( 4 bits)

B ==> The RAM word addressed by Badd is read into this variable. ( 4 bits)

Q ==> Used to represent the slave in the Q register. ( 4 bits)

F ==> ALU output . ( 4 bits)

*******************************************************

LOCAL SIGNALS (in VHDL concurrent blocks) :

## IN BLOCK "alu" :

R_ext ==> Extended input data for ALU. This consists of "RE"

concatenated with a '0' bit at the MSB position.

This extended ALU input is used to facilitate

computation of carry bit "C4". ( 5 bits)

( R_ext := '0' & RE; )

S_ext ==> Extended input data for ALU. This consists of "S"

concatenated with a '0' bit at the MSB position.

This extended ALU input is used to facilitate

computation of carry bit "C4". ( 5 bits)

( R_ext := '0' & S; )

result ==> Extended output from ALU. ( 5 bits)

( "F" is obtained from "result".

F <= result(3 downto 0); )

temp_p ==> An intermediate variable, calculated from "result" and used in the

computation of propagate term "Pbar". ( 5 bits)

temp_g ==> An intermediate variable, calculated from "result" and used in the

computation of generate term "Gbar". ( 5 bits)

-----------------------------------------------------------------

The model in file "funct_block_alg_beh2901.vhdl" contains detailed comments describing the

operation of each block. For further details, please refer to file

"am2901.doc".