--------------------------------------------------------------------------------

--

--   AM2901 Benchmark

--

-- Source: AMD data book

--

-- VHDL Benchmark author Indraneel Ghosh

--                       University Of California, Irvine, CA 92717

--

-- Developed on Nov 1, 1991

--

--------------------------------------------------------------------------------

The "mem" block (RAM) operation is controlled by three most significant bits

( I8, I7, I6) in the instruction. Thus there are eight possible cases.

The test vectors in "tv_mem.vhdl" test the "mem" component in

all modes of operation for each of the eight cases.

NOTE : The Aadd is just there for reading from memory locations.

----------------------------------------------------------

(1) Load F into RAM(Badd). Badd = 0

F <= "0111";

I <= "010000000";

RAM0 <= 'Z';

RAM3 <= 'Z';

Aadd <= 0;

Badd <= 0;

----------------------------------------------------------

(2) Load F into RAM(Badd) where Badd = 1

F <= "1001";

I <= "011000000";

RAM0 <= 'Z';

RAM3 <= 'Z';

Aadd <= 0;

Badd <= 1;

----------------------------------------------------------

(3) Do nothing

F <= "0000";

I <= "000000000";

RAM0 <= 'Z';

RAM3 <= 'Z';

Aadd <= 1;

Badd <= 0;

----------------------------------------------------------

(4) Do nothing

F <= "1111";

I <= "001000000";

RAM0 <= 'Z';

RAM3 <= 'Z';

Aadd <= 0;

Badd <= 1;

----------------------------------------------------------

(5)  Down shift F and load that into RAM(Badd)

with input 0.  Badd = 2.

F <= "1001";

I <= "100000000";

RAM0 <= 'Z';

RAM3 <= '0';

Aadd <= 1;

Badd <= 2;

----------------------------------------------------------

(6) Down shift F and load that into RAM(Badd)

with input 1.  Badd = 3

F <= "0111";

I <= "100000000";

RAM0 <= 'Z';

RAM3 <= '1';

Aadd <= 0;

Badd <= 3;

----------------------------------------------------------

(7) Down shift F and load that into RAM(Badd)

with input 0.  Badd = 4

F <= "1101";

I <= "101000000";

RAM0 <= 'Z';

RAM3 <= '0';

Aadd <= 2;

Badd <= 4;

----------------------------------------------------------

(8) Down shift F and load that into RAM(Badd)

with input 1.  Badd = 5

F <= "1101";

I <= "101000000";

RAM0 <= 'Z';

RAM3 <= '1';

Aadd <= 3;

Badd <= 5;

----------------------------------------------------------

(9) Up shift F and load that into RAM(Badd)

with input 0.  Badd = 2

F <= "0110";

I <= "110000000";

RAM0 <= '0';

RAM3 <= 'Z';

Aadd <= 4;

Badd <= 2;

----------------------------------------------------------

(10) Up shift F and load that into RAM(Badd)

with input 1.  Badd = 6

F <= "0000";

I <= "110000000";

RAM0 <= '1';

RAM3 <= 'Z';

Aadd <= 2;

Badd <= 6;

----------------------------------------------------------

(11) Up shift F and load that into RAM(Badd)

with input 0.  Badd = 7

F <= "1101";

I <= "111000000";

RAM0 <= '0';

RAM3 <= 'Z';

Aadd <= 5;

Badd <= 7;

----------------------------------------------------------

(12) Up shift F and load that into RAM(Badd)

with input 1.  Badd = 8

F <= "0010";

I <= "111000000";

RAM0 <= '1';

RAM3 <= 'Z';

Aadd <= 6;

Badd <= 8;

----------------------------------------------------------